;

;

; º This file is generated by Interactive DisAssembler (IDA)        º

; º Copyright (c) 1991-94 by Ilfak Guilfanov. (2:5020/209@fidonet)  º

;

;

0000

0000		      ;	Processor:	  Z80

0000		      ;	Target assembler: Table	Driven Assembler (TASM)	by Speech Technology Inc.

0000                 ; segment 'seg000'

0000

0000		      ;--------------------------------------------------------------------

0000 F3				      di

0001 3E01			      ld      a, 1

0003 D3FE			      out     (0FEh), a	      ;	BORDER =1 blue

0005 310090			      ld      sp, 9000h

0008		      ;	clearing pixels	screen #4000-#57FF

0008 210040			      ld      hl, 4000h

000B 3600			      ld      (hl), 0

000D 110140			      ld      de, 4001h

0010 01FF17			      ld      bc, 17FFh

0013 EDB0			      ldir

0015		      ;	setting	INK=7 PAPER=0 for screen #4000-#57FF

0015 23				      inc     hl

0016 13				      inc     de

0017 3638                           ld      (hl), 38h ; '8' ; <void>

0019 01FF02			      ld      bc, 2FFh ; <void>

001C EDB0			      ldir

001E		      ;	clearing pixels	2nd screen #6000-#71FF

001E 210060			      ld      hl, 6000h

0021 3600			      ld      (hl), 0

0023 110160			      ld      de, 6001h

0026 01FF17			      ld      bc, 17FFh

0029 EDB0			      ldir

002B		      ;	setting	INK=7 PAPER=0 for 2nd screen

002B 23				      inc     hl

002C 13				      inc     de

002D 3638                           ld      (hl), 38h ; '8' ; <void>

002F 01FF02			      ld      bc, 2FFh ; <void>

0032 EDB0			      ldir

0034		      ------------

0034 3E00			      ld      a, 0

0036 D3E7			      out     (0E7h), a	      ;	Reset 8272 control register.

0038 3E10			      ld      a, 10h

003A D3E7			      out     (0E7h), a	      ;	Set signal RES8272

003C CD3002			      call    Init8272

003F		      ------------ ; <void>

003F 216602			      ld      hl, 266h ; <void>; copy 19 octets	from #0266 to #9010

0042 111090			      ld      de, 9010h

0045 011300			      ld      bc, 13h

0048 EDB0			      ldir

004A		      ------------

004A 011090			      ld      bc, 9010h

004D CD5C01			      call    loc_15C

0050 3E02			      ld      a, 2

0052 D3FE			      out     (0FEh), a	      ;	BORDER=2 red

0054 3E08			      ld      a, 8

0056 321B90			      ld      (byte_901B), a

0059 011090			      ld      bc, 9010h

005C CD5C01			      call    loc_15C

005F 384C			      jr      c, loc_AD

0061 3E03			      ld      a, 3

0063 D3FE			      out     (0FEh), a	      ;	BORDER=3 cyan

0065 3E01			      ld      a, 1

0067 321B90			      ld      (byte_901B), a

006A 3A0E90			      ld      a, (byte_900E)

006D 321290			      ld      (byte_9012), a

0070 FE02			      cp      2

0072 2821			      jr      z, loc_95

0074

0074		      loc_74:

0074							      ;	CODE XREF: 007Aj

0074 011090			      ld      bc, 9010h

0077 CD5C01			      call    loc_15C

007A 38F8			      jr      c, loc_74

007C 3E04			      ld      a, 4

007E D3FE			      out     (0FEh), a	      ;	BORDER=4 green

0080 FD3406			      inc     (iy+6)

0083 FD3404			      inc     (iy+4)

0086

0086		      loc_86:

0086							      ;	CODE XREF: 008Cj

0086 011090			      ld      bc, 9010h

0089 CD5C01			      call    loc_15C

008C 38F8			      jr      c, loc_86

008E 3E00			      ld      a, 0

0090 D3FE			      out     (0FEh), a	      ;	BORDER=0 black

0092 C30080			      jp      unk_8000

0095		      ;--------------------------------------------------------------------

0095

0095		      loc_95:

0095							      ;	CODE XREF: 0072j

0095 3E05			      ld      a, 5

0097 D3FE			      out     (0FEh), a	      ;	BORDER=5 magenta

0099 3E00			      ld      a, 0

009B 321490			      ld      (byte_9014), a

009E

009E		      loc_9E:

009E							      ;	CODE XREF: 00A4j

009E 011090			      ld      bc, 9010h

00A1 CD5C01			      call    loc_15C

00A4 38F8			      jr      c, loc_9E

00A6 3E00			      ld      a, 0

00A8 D3FE			      out     (0FEh), a

00AA C30080			      jp      unk_8000

00AD		      ;--------------------------------------------------------------------

00AD

00AD		      loc_AD:

00AD							      ;	CODE XREF: 005Fj

00AD 3E10			      ld      a, 10h

00AF D3E7			      out     (0E7h), a

00B1 76				      halt

00B2

00B2		      Set_8272_command:

00B2							      ;	CODE XREF: 00CBp

00B2 D3DF			      out     (0DFh), a	      ;	load command in	8272 data register

00B4 E3				      ex      (sp), hl

00B5 E3				      ex      (sp), hl

00B6 E3				      ex      (sp), hl

00B7 E3				      ex      (sp), hl

00B8 C9				      ret

00B9		      ;--------------------------------------------------------------------

00B9

00B9		      loc_B9:

00B9							      ;	CODE XREF: 0155p

00B9 DBDF			      in      a, (0DFh)

00BB E3				      ex      (sp), hl

00BC E3				      ex      (sp), hl

00BD E3				      ex      (sp), hl

00BE E3				      ex      (sp), hl

00BF C9				      ret

00C0		      ;--------------------------------------------------------------------

00C0

00C0		      loc_C0:

00C0							      ;	CODE XREF: 00DEp 00E4p 01BCp 01E9p

00C0							      ;	0202p 0217p 0220p

00C0 210090			      ld      hl, 9000h

00C3

00C3		      Loop_2:

00C3							      ;	CODE XREF: 00C6j 00D0j 023Ep

00C3 DBDD			      in      a, (0DDh)	      ;	8272 GetStatus

00C5 17				      rla

00C6 30FB			      jr      nc, Loop_2      ;	RQM=0 not ready	then check again!

00C8 17				      rla

00C9 D8				      ret     c		      ;	RQM=1 DIO=1 so return

00CA		      RQM=1 DIO=0 => 8272 ready	to receive command

00CA 79				      ld      a, c

00CB CDB200			      call    Set_8272_command

00CE 4E				      ld      c, (hl)

00CF 23				      inc     hl

00D0 10F1			      djnz    Loop_2

00D2 C9				      ret

00D3		      ;--------------------------------------------------------------------

00D3

00D3		      loc_D3:

00D3							      ;	CODE XREF: 01ABp 01F2p

00D3 320190			      ld      (byte_9001), a

00D6 210F90			      ld      hl, 900Fh

00D9 BE				      cp      (hl)

00DA C8				      ret     z

00DB 010F03			      ld      bc, 30Fh ; <void>

00DE CDC000			      call    loc_C0

00E1		       ; <void>

00E1		      loc_E1: ;	<void>

00E1							      ;	CODE XREF: 00EFj 01ECp 021Ap 0223p ; <void>

00E1 010801			      ld      bc, 108h ; <void>

00E4 CDC000			      call    loc_C0

00E7 CD4001			      call    loc_140

00EA 3A0890			      ld      a, (byte_9008)

00ED E620                             and     20h ; ' ' ; <void>

00EF 28F0			      jr      z, loc_E1

00F1 210F90			      ld      hl, 900Fh

00F4 3A0990			      ld      a, (byte_9009)

00F7 77				      ld      (hl), a

00F8 3E0A			      ld      a, 0Ah

00FA		       ; <void>

00FA		      Loop_4: ;	<void>

00FA							      ;	CODE XREF: 0100j 0245p ; <void>

00FA 0EDA                             ld      c, 0DAh ; 'Ú' ; <void>

00FC

00FC		      Loop_3:

00FC							      ;	CODE XREF: 00FDj

00FC 0D				      dec     c

00FD 20FD			      jr      nz, Loop_3

00FF 3D				      dec     a

0100 20F8			      jr      nz, Loop_4

0102 C9				      ret

0103		      ;-------------------------------------------------------------------- ; <void>

0103		       ; <void>

0103		      loc_103: ; <void>

0103							      ;	CODE XREF: 01CBp ; <void>

0103 01DF00                           ld      bc, 0DFh ; 'ß' ; <void>

0106 110000			      ld      de, 0

0109

0109		      Loop_5:

0109							      ;	CODE XREF: 010Fj 0112j

0109 DBDD			      in      a, (0DDh)

010B 07				      rlca

010C 3812			      jr      c, loc_120

010E 1C				      inc     e

010F 20F8			      jr      nz, Loop_5

0111 14				      inc     d

0112 20F5			      jr      nz, Loop_5

0114

0114		      loc_114:

0114							      ;	CODE XREF: 0151j

0114 CD3002			      call    Init8272

0117 210890			      ld      hl, 9008h

011A 3640                             ld      (hl), 40h ; '@' ; <void>; Variable double from #9008 = #4040

011C 23				      inc     hl

011D 3640                             ld      (hl), 40h ; '@' ; <void>

011F C9				      ret

0120		      ;-------------------------------------------------------------------- ; <void>

0120		       ; <void>

0120		      loc_120: ; <void>

0120							      ;	CODE XREF: 010Cj ; <void>

0120 E640                             and     40h ; '@' ; <void>

0122 281C			      jr      z, loc_140

0124 EDA2			      ini

0126 E3				      ex      (sp), hl

0127 E3				      ex      (sp), hl

0128

0128		      Loop_6:

0128							      ;	CODE XREF: 012Bj 0131j 0136j

0128 DBDD			      in      a, (0DDh)

012A 07				      rlca

012B 30FB			      jr      nc, Loop_6

012D EDA2			      ini

012F E3				      ex      (sp), hl

0130 E3				      ex      (sp), hl

0131 20F5			      jr      nz, Loop_6

0133 FD350D			      dec     (iy+13)

0136 20F0			      jr      nz, Loop_6

0138 3E1B			      ld      a, 1Bh

013A D3E7			      out     (0E7h), a

013C E6FE                             and     0FEh ; 'þ' ; <void>

013E D3E7			      out     (0E7h), a

0140

0140		      loc_140:

0140							      ;	CODE XREF: 00E7p 0122j 0205p

0140 210890			      ld      hl, 9008h

0143 110000			      ld      de, 0

0146

0146		      NotReady:

0146							      ;	CODE XREF: 014Cj 014Fj 015Aj

0146 DBDD			      in      a, (0DDh)	      ;	8272 GetStatus

0148 17				      rla

0149 3808			      jr      c, READY_8272   ;	Jump to	8272_READY

014B 1C				      inc     e

014C 20F8			      jr      nz, NotReady

014E 14				      inc     d

014F 20F5			      jr      nz, NotReady

0151 18C1			      jr      loc_114	      ;	Timeout	waiting	for 8272_READY

0153		      ;--------------------------------------------------------------------

0153

0153		      READY_8272:

0153							      ;	CODE XREF: 0149j

0153 17				      rla

0153		      ;	End of subroutine **********

0153

0154 D0				      ret     nc

0155 CDB900			      call    loc_B9

0158 77				      ld      (hl), a

0159 23				      inc     hl

015A 18EA			      jr      NotReady

015C		      ;--------------------------------------------------------------------

015C

015C		      loc_15C:

015C							      ;	CODE XREF: 004Dp 005Cp 0077p 0089p

015C							      ;	00A1p

015C C5				      push    bc

015D FDE1			      pop     iy

015F FD361003			      ld      (iy+16), 3

0163 FD361104			      ld      (iy+17), 4

0167 AF				      xor     a

0168 320090			      ld      (byte_9000), a

016B 47				      ld      b, a

016C FD7E04			      ld      a, (iy+4)

016F FE10			      cp      10h

0171 3804			      jr      c, Next_1

0173 0604			      ld      b, 4

0175 D610			      sub     10h

0177

0177		      Next_1:

0177							      ;	CODE XREF: 0171j

0177 C5				      push    bc

0178 F5				      push    af

0179 FD7E02			      ld      a, (iy+2)

017C 3D				      dec     a

017D 212802			      ld      hl, 228h ; <void>

0180 114D02			      ld      de, 24Dh ; <void>

0183 2806			      jr      z, Next_2

0185 212C02			      ld      hl, 22Ch ; <void>

0188 115D02			      ld      de, 25Dh ; <void>

018B

018B		      Next_2:

018B							      ;	CODE XREF: 0183j

018B F1				      pop     af

018C 83				      add     a, e

018D 5F				      ld      e, a

018E 1A				      ld      a, (de)

018F 320390			      ld      (byte_9003), a

0192 110490			      ld      de, 9004h

0195 010400			      ld      bc, 4

0198 EDB0			      ldir

019A C1				      pop     bc

019B 3A0090			      ld      a, (byte_9000)

019E B0				      or      b

019F 320090			      ld      (byte_9000), a

01A2 78				      ld      a, b

01A3 0F				      rrca

01A4 0F				      rrca

01A5 320290			      ld      (byte_9002), a

01A8 FD7E03			      ld      a, (iy+3)

01AB CDD300			      call    loc_D3

01AE FD7E0B			      ld      a, (iy+11)

01B1 FE08			      cp      8

01B3 284A			      jr      z, loc_1FF

01B5 FE10			      cp      10h

01B7 285B			      jr      z, loc_214

01B9

01B9		      loc_1B9:

01B9							      ;	CODE XREF: 01D8j 01F5j

01B9 014609			      ld      bc, 946h

01BC CDC000			      call    loc_C0

01BF FD6E05			      .db 0FDh,6Eh,5 ; <BAD> ld	     yl, (iy+5)

01C2 FD6606			      .db 0FDh,66h,6 ; <BAD> ld	     yh, (iy+6)

01C5 FD7E02			      ld      a, (iy+2)

01C8 FD770D			      ld      (iy+13), a

01CB CD0301			      call    loc_103

01CE 3A0890			      ld      a, (byte_9008)

01D1 E6C0                             and     0C0h ; 'À' ; <void>

01D3 2822			      jr      z, loc_1F7

01D5 FD3511			      dec     (iy+17)

01D8 20DF			      jr      nz, loc_1B9

01DA FD3510			      dec     (iy+16)

01DD 3A0990			      ld      a, (byte_9009)

01E0 2818			      jr      z, loc_1F8+2

01E2 FD361104			      ld      (iy+17), 4

01E6 010702			      ld      bc, 207h ; <void>

01E9 CDC000			      call    loc_C0

01EC CDE100			      call    loc_E1

01EF FD7E03			      ld      a, (iy+3)

01F2 CDD300			      call    loc_D3

01F5 18C2			      jr      loc_1B9

01F7		      ;--------------------------------------------------------------------

01F7

01F7		      loc_1F7:

01F7							      ;	CODE XREF: 01D3j 0212j 0226j

01F7 AF				      xor     a

01F8

01F8		      loc_1F8:

01F8							      ;	CODE XREF: 01E0j 0210j

01F8 01C137			      ld      bc, 37C1h

01FB FD770C			      ld      (iy+12), a

01FE C9				      ret

01FF		      ;-------------------------------------------------------------------- ; <void>

01FF		       ; <void>

01FF		      loc_1FF: ; <void>

01FF							      ;	CODE XREF: 01B3j ; <void>

01FF 014A02			      ld      bc, 24Ah ; <void>

0202 CDC000			      call    loc_C0

0205 CD4001			      call    loc_140

0208 3A0890			      ld      a, (byte_9008)

020B E6C0                             and     0C0h ; 'À' ; <void>

020D 3A0990			      ld      a, (byte_9009)

0210 20E8			      jr      nz, loc_1F8+2

0212 18E3			      jr      loc_1F7

0214		      ;-------------------------------------------------------------------- ; <void>

0214		       ; <void>

0214		      loc_214: ; <void>

0214							      ;	CODE XREF: 01B7j ; <void>

0214 010702			      ld      bc, 207h ; <void>

0217 CDC000			      call    loc_C0

021A CDE100			      call    loc_E1

021D 010702			      ld      bc, 207h ; <void>

0220 CDC000			      call    loc_C0

0223 CDE100			      call    loc_E1

0226 18CF			      jr      loc_1F7

0226		      ;--------------------------------------------------------------------

0228 01				      .db    1 ;

0229 10				      .db  10h ;

022A 0E				      .db  0Eh ;

022B FF				      .db 0FFh ; ÿ

022C 02				      .db    2 ;

022D 09				      .db    9 ;

022E 1B				      .db  1Bh ;

022F FF				      .db 0FFh ; ÿ

0230		      ;--------------------------------------------------------------------

0230

0230		      Init8272:

0230							      ;	CODE XREF: 003Cp 0114p

0230 3E00			      ld      a, 0

0232 D3E7			      out     (0E7h), a	      ;	Reset 8272 external control register

0234 3E1A			      ld      a, 1Ah

0236 D3E7			      out     (0E7h), a	      ;	clear N555 and set DS0

0238 010303			      ld      bc, 303h ; <void>

023B 214B02			      ld      hl, 24Bh ; <void>

023E CDC300			      call    Loop_2

0241 0664                             ld      b, 64h ; 'd' ; <void>

0243

0243		      Loop_1:

0243							      ;	CODE XREF: 0248j

0243 3E0A			      ld      a, 0Ah

0245 CDFA00			      call    Loop_4

0248 10F9			      djnz    Loop_1

024A C9				      ret

024A		      ;--------------------------------------------------------------------

024B EF				      .db 0EFh ; ï

024C 01				      .db    1 ;

024D 01				      .db    1 ;

024E 03				      .db    3 ;

024F 05				      .db    5 ;

0250 07				      .db    7 ;

0251 09				      .db    9 ;

0252 0B				      .db  0Bh ;

0253 0D				      .db  0Dh ;

0254 0F				      .db  0Fh ;

0255 02				      .db    2 ;

0256 04				      .db    4 ;

0257 06				      .db    6 ;

0258 08				      .db    8 ;

0259 0A				      .db  0Ah ;

025A 0C				      .db  0Ch ;

025B 0E				      .db  0Eh ;

025C 10				      .db  10h ;

025D 01				      .db    1 ;

025E 03				      .db    3 ;

025F 05				      .db    5 ;

0260 07				      .db    7 ;

0261 09				      .db    9 ;

0262 02				      .db    2 ;

0263 04				      .db    4 ;

0264 06				      .db    6 ;

0265 08				      .db    8 ;

0266 01				      .db    1 ;	      ;	copied to #9010

0267 00				      .db    0 ;

0268 00				      .db    0 ;

0269 00				      .db    0 ;

026A 10				      .db  10h ;

026B 00				      .db    0 ;

026C 80				      .db  80h ; €

026D 00				      .db    0 ;

026E 00				      .db    0 ;

026F 00				      .db    0 ;

0270 00				      .db    0 ;

0271 10				      .db  10h ;

0272 00				      .db    0 ;

0273 00				      .db    0 ;

0274 00				      .db    0 ;

0275 00				      .db    0 ;

0276 00				      .db    0 ;

0277 00				      .db    0 ;

0278 00				      .db    0 ;

0279 00				      .db    0 ;

027A 00				      .db    0 ;

027B 00				      .db    0 ;

027C 00				      .db    0 ;

027D 00				      .db    0 ;

027E 00				      .db    0 ;

027F 00				      .db    0 ;

8000 00		      unk_8000:	      .block 1;	uninited data

8001 00				      .block 1;	uninited data

8002 00				      .block 1;	uninited data

8003 00				      .block 1;	uninited data

8004 00				      .block 1;	uninited data

8005 00				      .block 1;	uninited data

8006 00				      .block 1;	uninited data

8007 00				      .block 1;	uninited data

8008 00				      .block 1;	uninited data

8009 00				      .block 1;	uninited data

800A 00				      .block 1;	uninited data

800B 00				      .block 1;	uninited data

800C 00				      .block 1;	uninited data

800D 00				      .block 1;	uninited data

800E 00				      .block 1;	uninited data

800F 00				      .block 1;	uninited data

8010 00				      .block 1;	uninited data

8011 00				      .block 1;	uninited data

9000 00		      byte_9000:      .block 1

9001 00		      byte_9001:      .block 1

9002 00		      byte_9002:      .block 1

9003 00		      byte_9003:      .block 1

9004 00				      .block 1;	uninited data

9005 00				      .block 1;	uninited data

9006 00				      .block 1;	uninited data

9007 00				      .block 1;	uninited data

9008 00		      byte_9008:      .block 1

9009 00		      byte_9009:      .block 1

900A 00				      .block 1;	uninited data

900B 00				      .block 1;	uninited data

900C 00				      .block 1;	uninited data

900D 00				      .block 1;	uninited data

900E 00		      byte_900E:      .block 1

900F 00				      .block 1;	uninited data

9010 00				      .block 1;	uninited data

9011 00				      .block 1;	uninited data

9012 00		      byte_9012:      .block 1

9013 00				      .block 1

9014 00		      byte_9014:      .block 1

9015 00				      .block 1;	uninited data

9016 00				      .block 1;	uninited data

9017 00				      .block 1;	uninited data

9018 00				      .block 1;	uninited data

9019 00				      .block 1;	uninited data

901A 00				      .block 1;	uninited data

901B 00		      byte_901B:      .block 1

901C 00				      .block 1;	uninited data

901D 00				      .block 1;	uninited data

901E 00				      .block 1;	uninited data

901F 00				      .block 1;	uninited data

9020 00				      .block 1;	uninited data

9021 00				      .block 1;	uninited data

9022 00				      .block 1;	uninited data

9023 00				      .block 1;	uninited data

9024 00				      .block 1;	uninited data

9025 00				      .block 1;	uninited data

9026 00				      .block 1;	uninited data

9027 00				      .block 1;	uninited data

9028 00				      .block 1;	uninited data

9029 00				      .block 1;	uninited data

902A 00				      .block 1;	uninited data

902B 00				      .block 1;	uninited data

902C 00				      .block 1;	uninited data

902D 00				      .block 1;	uninited data

902E 00				      .block 1;	uninited data

902F 00				      .block 1;	uninited data

9030 00				      .block 1;	uninited data

9031 00				      .block 1;	uninited data

9032 00				      .block 1;	uninited data

9033 00				      .block 1;	uninited data

9034 00				      .block 1;	uninited data

9035 00				      .block 1;	uninited data

9036 00				      .block 1;	uninited data

9037 00				      .block 1;	uninited data

9038 00				      .block 1;	uninited data

9039 00				      .block 1;	uninited data

903A 00				      .block 1;	uninited data

903B 00				      .block 1;	uninited data

903C 00				      .block 1;	uninited data

903D 00				      .block 1;	uninited data

903E 00				      .block 1;	uninited data

903F 00				      .block 1;	uninited data

9040 00				      .block 1;	uninited data

9041 00				      .block 1;	uninited data

9042 00				      .block 1;	uninited data

9043 00				      .block 1;	uninited data

9044 00				      .block 1;	uninited data

9045 00				      .block 1;	uninited data

9046 00				      .block 1;	uninited data

9047 00				      .block 1;	uninited data

9048 00				      .block 1;	uninited data

9049 00				      .block 1;	uninited data

904A 00				      .block 1;	uninited data

904B 00				      .block 1;	uninited data

904C 00				      .block 1;	uninited data

904D 00				      .block 1;	uninited data

904E 00				      .block 1;	uninited data

904F 00				      .block 1;	uninited data

9050 00				      .block 1;	uninited data

9051 00				      .block 1;	uninited data

9052 00				      .block 1;	uninited data

9053 00				      .block 1;	uninited data

9054 00				      .block 1;	uninited data

9055 00				      .block 1;	uninited data

9056 00				      .block 1;	uninited data

9057 00				      .block 1;	uninited data

9058 00				      .block 1;	uninited data

9059 00				      .block 1;	uninited data

905A 00				      .block 1;	uninited data

905B 00				      .block 1;	uninited data

905C 00				      .block 1;	uninited data

905D 00				      .block 1;	uninited data

905E 00				      .block 1;	uninited data

905F 00				      .block 1;	uninited data

9060 00				      .block 1;	uninited data

9061 00				      .block 1;	uninited data

9062 00				      .block 1;	uninited data

9063 00				      .block 1;	uninited data

9064 00				      .block 1;	uninited data

9065 00				      .block 1;	uninited data

9066 00				      .block 1;	uninited data

9067 00				      .block 1;	uninited data

9068 00				      .block 1;	uninited data

9069 00				      .block 1;	uninited data

906A 00				      .block 1;	uninited data

906B 00				      .block 1;	uninited data

906C 00				      .block 1;	uninited data

906D 00				      .block 1;	uninited data

906E 00				      .block 1;	uninited data

FFFC 00				      .block 1;	uninited data

FFFD 00				      .block 1;	uninited data

FFFE 00				      .block 1;	uninited data

FFFE

FFFE                  ; end of 'seg000'

FFFE

FFFE

FFFE				      .end

CMD register

| Bit | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | n/u | n/u | MOD0 | MOD | MOD1 | SCR | LOCK | LOAD\ |

OutCMD\ = M1 + IORQ\ + A7\ + RD + A6 + A0\

| Addr line | A7 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 1 | 0 | x | x | x | x | x | 1 |

LOAD\ = Validare ROM in spatiul #0000-#3FFF (Q1). Inhibare selectie IF1.

LOCK\ = Inhibare scriere in spatiul #0000-#3FFF (Q1) atunci cand nu se activeaza IF1.

SCR   = Setat plaseaza memoria video in Q3 iar resetat in Q1. Numai cand MOD??????

MOD1  = VRAM (16k) mapat peste RAM (64k) indiferent de adresa.

MOD   = Setat in modul HiRes (80 coloane pentru CP/M) iar resetat in modul LoRes (32 coloane pentru BASIC)

MOD0  = Mapeaza in Q1 RAM in loc de VRAM.

Porturi:

E7 floppy drive interface control register ( out: TC, RES8272, DS0, DS1; in: bit0 only)

EF

F7

#FE  register for standard Sinclair peripherals (KBD, EAR, MIC, BELL, BORDER)

#8(9AB)F(137) memory configuration register CMD

#DF 8272 data register (R/W commands & data)

#DD 8272 status register (ReadOnly).